CONSTRUCTION TRAFFIC CONTROL INSTALLATION

ITEM 1103 – CONSTRUCTION TRAFFIC CONTROL

ON PAGE 6 OF SECTION 1103 OF THE ISPWC, PART 3 – WORKMANSHIP, SUBSECTION 3.1 - GENERAL, add the following:

- Temporary work zone traffic control (TC) equipment placed in public right-of-way under Ada County Highway District (ACHD) jurisdiction shall be placed in accordance with Section 6F.03 of the Manual on Uniform Traffic Control Devices (MUTCD) and as follows.

  - Ground mounted signs installed at the side of the road in rural areas shall be mounted at a height of at least 5 ft, measured from the bottom of the sign to the near edge of the pavement.  In business, commercial, and residential districts where parking and / or bicycle or pedestrian movement is likely to occur, or where there are other obstructions to sign visibility, or where there are two or more through lanes in each direction, the distance between the bottom of the sign and the near edge of the traveled way shall be at least 7 ft.

  - Neither portable nor permanent sign supports or barrels should be located on sidewalks, bicycle facilities, or areas designated for pedestrian or bicycle traffic, unless required for construction activities, in which case suitable detours must be provided.  Signs mounted lower than 7 ft should not project more than 4 inches into pedestrian facilities.

  - The height to the bottom of a secondary sign mounted below another sign may be 1 ft less than the appropriate height specified above.

  - All traffic control signs that will be left in place for longer than three (3) days shall be mounted on a wood or metal post set at least 30” into the ground or as directed by the Engineer.  Temporary installations will be allowed for signs left in place less than three (3) days.  All sign supports shall be crashworthy in accordance with NCHRP 350 standards.  Exceptions to this are the following signs from the MUTCD: R9-8 through R9-11a (Pedestrian and Sidewalk series), R11 (Road Closed series), W1-6 through W1-8 (Horizontal Arrow series), M4-10 (Horizontal Detour Arrow), or other similar types of signs that are typically mounted on portable barricades.

  - Signs mounted on barricades and barricade / sign combinations shall be crashworthy.

  - Signs mounted on barricades or other portable supports shall be no less than 1 ft above the traveled way.  Sign installations of this type shall only be allowed where approved by the Engineer.

  - Signs mounted on barricades shall not cover more than 50% of the top two rails or 33% of the total area of the three rails.

  - Large signs having an area exceeding 50 square feet that are installed on multiple crashworthy posts shall be mounted a minimum of 7 ft above the ground.

  - Temporary work zone traffic control signs that are not needed at the end of the work day are to be covered, turned, or removed from the work site.  Signs that are covered or turned shall be delineated by the use of reflective tape, cones, or barrels.  Signs mounted on portable supports are not to be rotated to a horizontal orientation; this creates a hazardous obstruction.