CONSTRUCTION TRAFFIC CONTROL ADMINISTRATION

All traffic control (TC) signing and detours are to be in place and approved by the engineer prior to the Contractor starting work.  All signing and channelization shall be per the Manual of Uniform Traffic Control Devices (MUTCD), latest edition, and Section 1103 of the Idaho Standards for Public Works Construction (ISPWC).  The engineer or his representative will be the sole judge in determining the acceptability of the condition and appearance of the traffic control and work zone devices.  Devices or signs determined to be in unacceptable condition are to be promptly replaced with materials of acceptable condition and appearance.

All stop and street name signs will remain installed and visible at their current location at all times.  Temporary stop and street name signs shall be provided for traffic control while the permanent signs are being replaced, relocated, or are obstructed.

The contractor may be required as part of this item to install and move the traffic control drums numerous times as needed to properly control traffic on the project.