ADA DURING CONSTRUCTION

When there is no direct impact to pedestrian facilities and the work is less than half of one day

The needs and control of all road users (motorists, bicyclists, and pedestrians within the highway and/or public right-of-way, including persons with disabilities in accordance with the Americans with Disabilities Act of 1990 (ADA), Title II, Paragraph 35.130) through a temporary traffic control “TTC” zone shall be an essential part of highway construction, utility work, maintenance operations, and the management of traffic incidents.  The primary function of TTC is to provide for the reasonably safe and efficient movement of road users through or around TTC zones while reasonably protecting workers, responders to traffic incidents, and equipment.

Temporary facilities, including reasonably safe pedestrian routes around work sites, are also covered by the accessibility requirements of the Americans with Disabilities Act of 1990 (ADA) (Public Law 101-336, 104 Stat.327, July 26, 1990. 42 USC 12101-12213 (as amended)).  TTC plans and devices shall be the responsibility of the contractor performing the construction, alteration and/or maintenance of the highway or public right-of-way.  When an existing continuous sidewalk or street crossing route cannot be maintained for pedestrians because of construction, either temporary walkways with curb ramps are to be provided, or the construction shall be phased to maintain access to the affected addresses.  Contractors shall be allowed flexibility as long as the requirements are met.

Contractors shall not block temporary walkways with contractor parking, materials piles, signs, rubble or rubbish.  Construction equipment and equipment operation must be separated from the temporary walkways.  At work zones where higher volumes of pedestrian traffic or school children exist, pedestrian fences or other protective barriers may be needed to prevent access into the construction area.